# 🧮 GCD Calculator Using Euclidean Algorithm in Verilog

## 📌 1. Introduction

The Greatest Common Divisor (GCD) of two numbers is the largest integer that divides both numbers exactly. It is widely used in:

🔹 Number theory 🔹 Cryptography 🔹 Digital hardware design

This document explores the theoretical aspects of designing a GCD Calculator using the Euclidean Algorithm in Verilog HDL.

## 📖 2. Understanding GCD

Given two positive integers A and B, the GCD(A, B) is the largest integer D such that: ✅ D divides A ✅ D divides B

### 📌 Examples:

- 🟢 GCD(12, 8) = 4

- 🟢 GCD(9, 6) = 3

- 🟢 GCD(15, 5) = 5

The most efficient way to compute GCD is through the Euclidean Algorithm.

## 🔄 3. Euclidean Algorithm for GCD

The Euclidean Algorithm works by repeatedly subtracting the smaller number from the larger number until both numbers become equal.

### ⚙️ Algorithm Steps:

1️⃣ Start with two numbers A and B. 2️⃣ If A = B, then GCD = A (or B). 3️⃣ If A > B, replace A with A - B. 4️⃣ If B > A, replace B with B - A. 5️⃣ Repeat steps 2-4 until A = B. 6️⃣ The final value of A (or B) is the GCD.

### 📊 Example Calculation:

#### 🔹 Find GCD(12, 8)

| Step | 🅰️ A | 🅱️ B | 🔄 Action |
| --- | --- | --- | --- |
| 1 | 12 | 8 | A > B → A = 12 - 8 = 4 |
| 2 | 4 | 8 | B > A → B = 8 - 4 = 4 |
| 3 | 4 | 4 | A = B → ✅ GCD = 4 |

#### 🔹 Find GCD(9, 6)

| Step | 🅰️ A | 🅱️ B | 🔄 Action |
| --- | --- | --- | --- |
| 1 | 9 | 6 | A > B → A = 9 - 6 = 3 |
| 2 | 3 | 6 | B > A → B = 6 - 3 = 3 |
| 3 | 3 | 3 | A = B → ✅ GCD = 3 |

## 🏗️ 4. Hardware Implementation Strategy

To implement the GCD Calculator in Verilog, we use a Finite State Machine (FSM) with three states:

🔹 State 1: IDLE 🕒

- Waits for the start signal.

- Loads the inputs A and B into internal registers.

🔹 State 2: COMPUTE 🔄

- If A > B, subtract B from A.

- If B > A, subtract A from B.

- Repeat the process until A = B.

🔹 State 3: DONE ✅

- The computed GCD is stored in the output register.

- The done signal is set high to indicate completion.

## ⚡ 5. Hardware Design Considerations

🔹 Bit-Width Selection: Supports 4-bit numbers, extendable to 8-bit or 16-bit. 🔹 Clock & Reset Handling: The FSM operates on a clock signal, and a reset signal reinitializes the system. 🔹 Optimization: A Modulo-based Euclidean Algorithm can be used for better efficiency. 🔹 Synthesis: The design can be implemented on FPGAs for real-time applications.

## 🎯 6. Applications of GCD Calculation in Digital Systems

✅ Cryptography: RSA encryption uses GCD for key generation. ✅ Signal Processing: Used in sampling rate conversion. ✅ Computer Networks: Used in checksum calculations and error detection. ✅ Mathematical Computation: Helps in simplifying fractions and number theory problems.

## 📌 7. Conclusion

The GCD Calculator is a fundamental module in digital hardware. The Euclidean Algorithm provides an efficient way to compute GCD. By using a state-machine approach in Verilog, we can design a reliable GCD calculator for FPGA or ASIC implementations. Future enhancements can include modulo-based computation and pipeline-based architectures for higher performance.

## 🚀 8. Future Improvements

🔹 Implementing Modulo-based Euclidean Algorithm. 🔹 Expanding bit-width support for 16-bit and 32-bit GCD calculations. 🔹 Using pipelining to compute GCD for multiple number pairs in parallel. 🔹 Implementing GCD in hardware accelerators for cryptographic applications.